Assigning IP Addresses:

- Subnet 0:

  - Router Interface: 1.1.0.129 / 255.255.255.192

  - PC0: 1.1.0.190 / 255.255.255.192 (Gateway: 1.1.0.129)

- Subnet 1:

  - Router Interface: 1.1.0.1 / 255.255.255.128

  - PC1: 1.1.0.126 / 255.255.255.128 (Gateway: 1.1.0.1)

  - PC3: 1.1.0.2 / 255.255.255.128 (Gateway: 1.1.0.1)

- Subnet 2:

  - Router Interface: 1.1.0.193 / 255.255.255.224

  - Web Server: 1.1.0.222 / 255.255.255.224 (Gateway: 1.1.0.193)

Network Topology Diagram:

- Label your network diagram with the appropriate IP addresses.

- (Draw or use Packet Tracer for visualization)

Configuration Commands:

Router (R0):

plaintext

Copy code

interface G0/0

ip address 1.1.0.129 255.255.255.192

ip ospf 1 area 0

no shutdown

interface Serial0/0/0

ip address 1.1.0.225 255.255.255.252

ip ospf 1 area 0

no shutdown

interface Serial0/0/1

ip address 1.1.0.230 255.255.255.252

ip ospf 1 area 0

no shutdown

router ospf 1

router-id 1.1.1.1

network 1.1.0.0 0.0.255.255 area 0

Router 1 (R1):

plaintext

Copy code

interface G0/0

ip address 1.1.0.1 255.255.255.128

ip ospf 1 area 0

no shutdown

interface Serial0/0/0

ip address 1.1.0.233 255.255.255.252

ip ospf 1 area 0

no shutdown

interface Serial0/0/1

ip address 1.1.0.226 255.255.255.252

ip ospf 1 area 0

no shutdown

router ospf 1

router-id 2.2.2.2

network 1.1.0.0 0.0.255.255 area 0

Router (R2):

plaintext

Copy code

interface G0/0

ip address 1.1.0.193 255.255.255.224

ip ospf 1 area 0

no shutdown

interface Serial0/0/0

ip address 1.1.0.229 255.255.255.252

ip ospf 1 area 0

no shutdown

interface Serial0/0/1

ip address 1.1.0.234 255.255.255.252

ip ospf 1 area 0

no shutdown

router ospf 1

router-id 3.3.3.3

network 1.1.0.0 0.0.255.255 area 0

Install Apache Web Server: 4. Install Apache on the designated Web Server in Subnet 2. http://apachelounge.com/download 5. Edit the index.htm file to add your full names. 6. Access the web server from PC1 using http://1.1.0.222 7. Activate httpd.exe on server bin/httpd.exe 8. Take a screenshot of the output and insert it below:

Task 1: ACL for PING from Subnet 0 9. ACL Statements R0:

plaintext

Copy code

access-list 100 permit icmp 1.1.0.128 0.0.0.63 any echo

access-list 101 permit icmp any 1.1.0.128 0.0.0.63 echo-reply

access-list 100 deny ip any any

- Deploy ACL on Router Interface R0:

plaintext

Copy code

int g0/0

ip access-group 100 in

- Check Your Work:

- Ping the web server in Subnet 2 from Subnet 0 (should succeed).

- Try connecting to the Apache server using a browser (should fail).

- Take screenshots of these outputs and insert them below:

Task 2: ACL for Web Server Access from PC1 12. ACL Statements R1:

plaintext

Copy code

access-list 101 permit tcp host 1.1.0.126 host 1.1.0.222 eq www

access-list 101 deny tcp host 1.1.0.2 host 1.1.0.222 eq www

access-list 101 permit ip any any

- Deploy ACL on Router Interface R1:

plaintext

Copy code

int g0/0

ip access-group 101 in

- Check Your Work:

- Access the server from PC1 using a web browser (should work).

- Ping the server from PC3 (should work).

- Access the server from PC3 using a web browser (should fail).

- Take screenshots of these outputs and insert them below:

Task 3: ACL for PING from First Half of Subnet 1 15. ACL Statements: On R0:

plaintext

Copy code

no access-list 100

access-list 100 permit icmp 1.1.0.0 0.0.0.63 1.1.0.128 0.0.0.127

access-list 100 deny icmp 1.1.0.64 0.0.0.63 1.1.0.128 0.0.0.127

access-list 100 permit ip any any

On R1:

plaintext

Copy code

access-list 100 permit icmp 1.1.0.0 0.0.0.63 1.1.0.128 0.0.0.63 echo

access-list 100 permit icmp 1.1.0.0 0.0.0.127 1.1.0.128 0.0.0.63 echo-reply

- Deploy ACL on Router Interface: On R0:

plaintext

Copy code

interface g0/0

ip access-group 100 in

On R1:

plaintext

Copy code

Int g0/0

ip access-group 100 in

- Check Your Work:

- Ping from PC1 to PC0 (must fail).

- Ping from PC3 to PC0 (must succeed).

- Ping from PC0 to PC1 and PC3 (must succeed).